- SECTION 02270 – TEMPORARY SOIL EROSION CONTROL

- PART 1 – GENERAL

  - GENERAL CONDITIONS:  The General Conditions and Special Conditions contained in this IFB shall govern this section of the work.

  - WORK INCLUDED:  Submit three (3) sets of the erosion control materials for approval by the Engineer.  Furnish all labor, materials, services, equipment and related items necessary to implement the temporary erosion control measures, submitted separately, as required by these specifications and as ordered by the Engineer during the life of the contract to control water pollution through the use of berms, dikes, dams, sediment basins, fiber mats, netting, gravel, mulches, grasses, slope drains, and other erosion control devices or methods.

- Temporary erosion and siltation control measures as described herein shall be applied to any erodible material within this project, including local material sources and work areas.

- The Contractor shall be responsible for providing the necessary erosion control measures which are shown on the plans or which may be ordered by the Engineer.  All grading operations shall be performed in conformance with the applicable provisions of the “Water Pollution Control and Water Quality Standards” contained in the “Public Health Regulations,” State Department of Health.

- The Contractor shall be responsible for promptly (next day after storms) removing all silt and debris resulting from his work and deposited in drainage facilities, roadways, neighboring lands, and other areas.

  - RELATED WORK IN OTHER SECTIONS

Fine Grading of Roadway Area	Section 02230

- PART 2 – PRODUCTS

  - MATERIALS

- Mulches:  To be bagasse, hay, straw, fiber mats, netting, wood cellulose, bark, wood chips, or other suitable material acceptable to the Engineer and shall be reasonably clean and free of noxious weeds and deleterious materials.

- Catch Basin Inlet Filters:  “True Dam” sediment filter (by Dandy Products, Inc.) or approved equal.

C.	Silt Fence Fabric:  Mirafi N140 or approved equal.

- PART 3 – EXECUTION

  - TEMPORARY EROSION CONTROL

- The Construction Manager has the authority to limit the surface area exposed by clearing and grubbing and to limit the surface area exposed by excavation, borrow and fill operations.  The Construction Manager may also direct the Contractor to provide immediate, permanent, or temporary pollution control measures to prevent contamination of streams, drainage channels and pipes, roads, neighboring lands, and other areas.

Except for specified measures which may be shown on the plans, the Contractor shall determine the appropriate erosion control measures to use.   Such work may involve the construction of temporary berms, dikes, dams, sediment basins, and slope drains, and the use of temporary mulches, mats, and grassing, or the construction and use of other control devices or methods as necessary to control erosion.

- The Contractor shall incorporate all erosion control measures shown in the plans.  The erosion controls may be modified as necessary to adjust to conditions that develop during construction.  All modifications are subject to approval by the Engineer.

- The Contractor shall limit the surface area exposed by grubbing, stripping of topsoil, and grading to that which is necessary for him to perform the next operation and which is within his capability and progress in keeping the finish grading, mulching, grassing, and other such pollution control measures current.

The grubbing of the vegetative root mat and stumps and the stripping of topsoil shall be confined within the limits of grading which can be actively and continuously prosecuted within 15 calendar days.  The area to be graded shall be limited to the minimum area necessary to accommodate the Contractor’s equipment and work force and shall not at any time exceed 15 acres, unless otherwise stated on plans, without prior approval of the Construction Manager.

Any area remaining bared or cleared for more than 10 calendar days and which is not within the limits of active construction shall be immediately hydro-mulch seeded or remedied as directed by the Engineer at the Contractor’s expense without cost to the Department.  All areas where finish grading has been completed shall be grassed within three calendar days after the completion of grading for that area.

- The Contractor shall, at the end of each work operation in any one day, shape the earthwork in such a manner as to control and direct the runoff to minimize the erosion of soils.  He shall construct earth berms along the top edges of embankments or along the property line with adjacent properties, streams and water channels, to intercept any runoff.  Temporary slope drains shall be provided to carry runoff from the top of cuts and fills.  Temporary facilities for controlled discharges shall be provided for runoff impounded, directed, or controlled by project activities or by any erosion control measure employed.

- The temporary erosion and siltation control measures outlined in these specifications are minimum requirements and shall not preclude the provision of any additional measures which the Contractor may deem necessary.  Damages caused by the erosion of soils and the pollution of downstream areas shall be the responsibility of the Contractor and all costs for repairing, correcting, replacing and cleaning damaged or polluted facilities shall be borne by the Contractor.

- END OF SECTION